#include <WiFi.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <DHT.h>

#include <WebServer.h>

// ================= WIFI =================

#define WIFI_SSID "Pavi"

#define WIFI_PASSWORD "14150000"

// ================= STATIC IP (IMPORTANT FIX) =================

IPAddress local_IP(10, 143, 124, 116);

IPAddress gateway(10, 143, 124, 1);

IPAddress subnet(255, 255, 255, 0);

// ================= LCD =================

LiquidCrystal_I2C lcd(0x27, 16, 2);

// ================= DHT22 =================

#define DHTPIN 4

#define DHTTYPE DHT22

DHT dht(DHTPIN, DHTTYPE);

// ================= WEB SERVER =================

WebServer server(80);

// ================= PINS =================

#define LED_PIN     15

#define BUZZER_PIN  25

#define RELAY_PIN   26

#define REED_PIN    27

// ================= GSM =================

#define GSM_RX 16

#define GSM_TX 17

HardwareSerial gsm(2);

// ================= THRESHOLDS =================

const float TEMP_MIN = 2.0;

const float TEMP_MAX = 8.0;

// ================= RELAY LOGIC =================

#define RELAY_ON  LOW

#define RELAY_OFF HIGH

// ================= DOOR LOGIC =================

#define DOOR_OPEN_STATE LOW

// ================= TIMERS =================

unsigned long lcdTimer = 0;

bool showDoorScreen = false;

// ================= GSM FLAGS =================

bool highSent = false;

bool lowSent = false;

bool doorSent = false;

// ================= USER PHONE =================

String phone = "+917674053681";

// ================= GSM FUNCTION =================

void sendSMS(String msg) {

gsm.println("AT");

delay(500);

gsm.println("AT+CMGF=1");

delay(500);

gsm.print("AT+CMGS=\"");

gsm.print(phone);

gsm.println("\"");

delay(500);

gsm.print(msg);

delay(500);

gsm.write(26);

delay(5000);

}

// ================= WIFI CONNECT (FIXED) =================

bool connectWiFi(unsigned long timeoutMs = 15000) {

WiFi.mode(WIFI_STA);

// 🔥 STATIC IP CONFIGURATION

if (!WiFi.config(local_IP, gateway, subnet)) {

Serial.println("STA Failed to configure static IP");

}

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

Serial.print("Connecting to WiFi");

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Connecting WiFi");

unsigned long startTime = millis();

while (WiFi.status() != WL_CONNECTED && millis() - startTime < timeoutMs) {

Serial.print(".");

delay(500);

}

if (WiFi.status() == WL_CONNECTED) {

Serial.println("\nWiFi Connected");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("WiFi Connected");

lcd.setCursor(0, 1);

lcd.print(WiFi.localIP().toString());

delay(1500);

return true;

}

Serial.println("\nWiFi Failed");

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("WiFi Failed");

lcd.setCursor(0, 1);

lcd.print("Offline Mode");

delay(1500);

return false;

}

// ================= API =================

void handleRoot() {

float temp = dht.readTemperature();

float hum = dht.readHumidity();

if (isnan(temp) || isnan(hum)) {

server.send(500, "application/json", "{\"error\":\"sensor\"}");

return;

}

String json = "{\"temp\":" + String(temp, 1) + ",\"hum\":" + String(hum, 0) + "}";

server.send(200, "application/json", json);

}

// ================= SETUP =================

void setup() {

Serial.begin(115200);

dht.begin();

lcd.init();

lcd.backlight();

pinMode(LED_PIN, OUTPUT);

pinMode(BUZZER_PIN, OUTPUT);

pinMode(RELAY_PIN, OUTPUT);

pinMode(REED_PIN, INPUT_PULLUP);

digitalWrite(LED_PIN, LOW);

digitalWrite(BUZZER_PIN, LOW);

digitalWrite(RELAY_PIN, RELAY_OFF);

gsm.begin(9600, SERIAL_8N1, GSM_RX, GSM_TX);

lcd.setCursor(0, 0);

lcd.print("Cold Chain Sys");

lcd.setCursor(0, 1);

lcd.print("Starting...");

delay(1500);

bool wifiOK = connectWiFi();

if (wifiOK) {

server.on("/", handleRoot);

server.on("/data", handleRoot);

server.begin();

Serial.println("HTTP Server Started");

}

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("System Ready");

delay(1000);

lcd.clear();

}

// ================= LOOP =================

void loop() {

server.handleClient();

float temp = dht.readTemperature();

float hum = dht.readHumidity();

bool doorOpen = (digitalRead(REED_PIN) == DOOR_OPEN_STATE);

if (isnan(temp) || isnan(hum)) {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Sensor Error!");

delay(2000);

return;

}

bool cooling = !(temp >= TEMP_MIN && temp <= TEMP_MAX);

digitalWrite(RELAY_PIN, cooling ? RELAY_ON : RELAY_OFF);

digitalWrite(LED_PIN, (cooling || doorOpen) ? HIGH : LOW);

digitalWrite(BUZZER_PIN, (cooling || doorOpen) ? HIGH : LOW);

if (millis() - lcdTimer > 2000) {

lcdTimer = millis();

showDoorScreen = !showDoorScreen;

}

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("T:");

lcd.print(temp, 1);

lcd.print(" H:");

lcd.print(hum, 0);

lcd.setCursor(0, 1);

lcd.print(showDoorScreen ? (doorOpen ? "Door OPEN" : "Door CLOSED")

: (cooling ? "Cooling ON" : "Normal"));

Serial.print("Temp: ");

Serial.print(temp);

Serial.print(" Hum: ");

Serial.println(hum);

// ================= GSM ALERTS =================

if (temp > TEMP_MAX && !highSent) {

sendSMS("High Temp Alert " + String(temp));

highSent = true;

lowSent = false;

}

else if (temp < TEMP_MIN && !lowSent) {

sendSMS("Low Temp Alert " + String(temp));

lowSent = true;

highSent = false;

}

else if (temp >= TEMP_MIN && temp <= TEMP_MAX) {

highSent = false;

lowSent = false;

}

if (doorOpen && !doorSent) {

sendSMS("Door Open Alert!");

doorSent = true;

}

else if (!doorOpen) {

doorSent = false;

}

delay(1000);

}