🔌 COMPLETE PIN CONNECTIONS (FINAL HARDWARE)

🔷 1. ESP32 (Main Controller)

This is the center of all connections.

🌡 2. DHT22 Sensor

Pin 1 (VCC)  → ESP32 3.3V  
Pin 2 (DATA) → ESP32 GPIO4  
Pin 4 (GND)  → ESP32 GND

👉 Add:

- 10kΩ resistor between VCC and DATA

📺 3. LCD 16x2 (I2C)

GND → ESP32 GND  
VCC → ESP32 VIN (5V)  
SDA → ESP32 GPIO21  
SCL → ESP32 GPIO22

🚪 4. Reed Sensor (Door Sensor)

One terminal → ESP32 GPIO27  
Other terminal → GND

👉 Uses internal pull-up

🔴 5. LED (Status Indicator)

ESP32 GPIO16 → 220Ω resistor → LED (+)  
LED (-) → GND

🔔 6. Buzzer

Buzzer + → ESP32 GPIO25  
Buzzer - → GND

❄️ 7. Peltier Module (VERY IMPORTANT)

👉 Use MOSFET (DO NOT connect directly)

Peltier + → 12V Power Supply (+)

Peltier - → MOSFET Drain  
MOSFET Source → GND  

ESP32 GPIO16 → 220Ω → MOSFET Gate  
10kΩ resistor → Gate to GND

🌬 8. Cooling Fan

Fan + → 12V Power  
Fan - → GND

👉 Connect parallel with Peltier

📡 9. GSM Module (SIM900)

Power:

VCC → 4V (Battery / LM2596 output)
GND → GND

Communication:

GSM TX → ESP32 GPIO16 (RX2)  
GSM RX → ESP32 GPIO17 (TX2)

⚡ 10. LM2596 Buck Converter

IN+ → 12V Supply  
IN- → GND  

OUT+ → 5V / 4V devices  
OUT- → GND

🔋 11. Battery Pack

Battery + → GSM VCC  
Battery - → GND

🔁 SYSTEM FLOW (VERY IMPORTANT)

DHT22 + Reed Sensor
        ↓
      ESP32
        ↓
 Decision Logic (2°C – 8°C)
        ↓
 LCD Display + LED + Buzzer
        ↓
 MOSFET → Peltier + Fan
        ↓
 GSM → SMS Alert